Build JEM0.9 based on JEM0. Basis should be the old de-serialisers, Spartan input stage.

- Simplify ref-clock distribution (crystal clock via input FPGA – check for potential problems with DLL position). Provide pads for filter capacitor. Use low strength outputs.  Replicate BX clock on the FPGA rather than on buffer.

- Run input synchronisation off a separate input line connected to bit0. Check for signal distortion beforehand (either short-circuit 2 inputs or use capacitor to simulate input capacitance).

- Convert to new terminators or omit them altogether, dependent  on test results

- Combine TTC interface and ROC in a common FPGA. Might have to be 300E.

- Omit buffer on TTCbus. Either drive p2p from FPGA for all input FPGAs or run true bus, source terminated (or current controlled) (one bus for inputs plus one for Main/ROC might be appropriate).

- Extend p2p links for inp-FPGA  ROC slightly, if possible.

Modifications to be considered only if easily accomplished:

- XC2S300E for input FPGA

- Improved VME (bus or p2p)

- FIO loop-through (only if impedance tests successful)

- XC2S600E for jet processor / sum processor