ATLAS Level-1 Calorimeter Trigger

Jet / Energy Processor Module

Programming Model

Version 1.2 (post PRR)

Date: 21 November 2006

Table of Contents

1	Memory Map	3

1.1	Mode	3

1.2	Sub Base Address	4

2	VME CPLD register map	4

2.1	MOD_ID_B	4

2.2	STATUS_REG	4

2.3	CFG_MASK_REG	4

2.4	FPGA_RESET_REG	4

3	SystemACE register map	5

4	Sum processor register map	5

4.1	STATUS_REG	5

4.2	CONTROL_REG	5

4.3	PULSE_REG	5

4.4	CFG_MASK_REG	6

4.5	FPGA_RESET_REG	6

4.6	DONE_REG	6

4.7	TTC_STATUS_REG	6

4.8	TTC_CONTROL_REG	6

4.9	READ_REQUEST_DELAY_REG	6

4.10	ROC_SLICE_REG	7

4.11	BC_PRESET_REG	7

4.12	SPY_MEM_EXY_PORT	7

4.13	RESET_SPY_MEM_EXY_COUNTER	7

4.14	SPY_MEM_ET_PORT	7

4.15	RESET_SPY_MEM_EXY_COUNTER	7

5	Input FPGA register map	7

5.1	PULSE_REG	8

5.2	CONTROL_REG	8

5.3	THRESHOLD_LOW , THRESHOLD_HIGH	8

5.4	PLAYSPY_REG	8

5.5	RESET_PLAYSPY_COUNTER	8

5.6	Channel Space	8

5.6.1	Ch_status	9

5.6.2	Ch_control	9

5.6.3	Ch_Link_err	9

5.6.4	Ch_Par_err	9

5.6.5	Ch_Test_Pat_err	9

5.6.6	Ch_mult	9

6	9

In the following a programming model for VME access to registers and memories of the JEM and its components is described. The description relates to the current FPGA firmware. The register model is likely to change for future firmware revisions. Each JEP crate will have a local CPU directly accessing the VME part of the crate backplane and thus an entirely self-contained VME address space. A suitable documentation of the programming model will be provided to accompany the final modules.

Guidelines

- The standard access is VME A24/D16.

- There are no write-only registers except for pulse and configuration registers. All registers can be read by the crate CPU via VME.

- The register bits generally have the same meaning for reads as for writes.

  - All Status Registers are read-only register

  - All Control Registers are read/write or pulse registers

  - Reading back a register will generally return the last value written

- Attempts to write to read-only registers or undefined portions of registers will not change the contents of the non-modifiable fields.

- Fully synchronous design protects all VME registers from being read via crate CPU while being altered by the JEM itself at the same time.

- When the address space occupied by the JEM is accessed, it will always respond with a handshake (DTACK*) to avoid a bus error.

- The power-up condition of all registers and BlockRAMs will be all zeros, unless otherwise stated.

Notation

- RO means that the computer can only read the value of this register; writing has no effect either to the value or the state of the module.

- RW means that the computer can affect the state of the module by writing to this register.

- PR means the register is a pulse register. Writing a ‘1’ triggers an action.

# Memory Map

In the following an overview of the address scheme for a JEP crate and its subsystems is given. Detailed register descriptions are given below. The jet processor programming model is so far available as a separate document only.

| Address Line | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use | 1 | JEM Base address |  |  |  | mode |  | FPGA sub address |  |  |  | Register address space for each FPGA |  |  |  |  |  |  |  |  |  |  |  |

## Mode

| VME Address (binary) | Device |
| --- | --- |
| 00 | VME registers |
| 10 | configuration |

Address bit 18 selects FPGA configuration download. The configuration space is defined as a write only block memory that returns 0xFFFF when read. The FPGAs selected for configuration are defined by bits 16 and 15 according to 1.2 .

## Sub Base Address

| VME Address (Hex) | Device |
| --- | --- |
| 0 | VME CPLD |
| 2 | SystemACE |
| 4 | Sum FPGA |
| 8 | JET FPGA |
| C | Input FPGA R |
| D | Input FPGA S |
| E | Input FPGA T |
| F | Input FPGA U |

# VME CPLD register map

| VME Address (Hex) | Type | Name | Description |
| --- | --- | --- | --- |
| 00 | RO | MOD_ID_A | Module ID A: module type |
| 02 | RO | MOD_ID_B | Serial number & Module Revision number |
| 04 | RO | VERSION_REG | Firmware version number |
| 06 | RO | STATUS_REG | Status register |
| 10 | RW | CFG_MASK_REG | Mask FPGAs for configuration |
| 12 | PR | FPGA_RESET_REG | Reset FPGA |

## MOD_ID_B

A register containing module identifiers.

| Bit | Description |
| --- | --- |
| 0 - 7 | Serial number |
| 8-15 | Revision number |

## STATUS_REG

A register containing module status information.

| Bit | Description |
| --- | --- |
| 0 | TTC clock available |
| 1 | Sum Processor Configuration done |

## CFG_MASK_REG

A register for enabling  FPGA to be configured via VME.

| Bit | Description |
| --- | --- |
| 0 | Configure Sum-FPGA |

## FPGA_RESET_REG

A register for resetting FPGA.

| Bit | Description |
| --- | --- |
| 0 | Reset Sum-FPGA |

# SystemACE register map

The SystemAce chip is mapped into the VME address space. For SystemACE registers see http://direct.xilinx.com/bvdocs/publications/ds080.pdf, p. 18. 16-bit wide access possible only. The registers are correctly mapped to even VME addresses.

# Sum processor register map

| VME Address (HEX) | Type | Name | Description |
| --- | --- | --- | --- |
| 00 | RO | VERSION_REG | Firmware version number |
| 02 | RO | STATUS_REG | Status Register |
| 04 | RW | CONTROL_REG | Control Register |
| 06 | PR | PULSE_REG | Pulse Register |
| 10 | RW | CFG_MASK_REG | Configuration mask |
| 12 | PR | FPGA_RESET_REG | FPGA reset |
| 14 | RO | DONE_REG | FPGA configuration status |
| 40 | RW | TTC_CONTROL_REG | I2C control register of TTCrx chip |
| 42 | RO | TTC_STATUS_REG | I2C status register of TTCrx chip |
| 60 | RW | READ_REQUEST_DELAY_REG | Delay read request signal |
| 62 | RW | ROC_SLICE_REG | Slice count register |
| 64 | RW | BC_PRESET_REG | Bunch counter correction |
| A0 | RO | SPY_MEM_EXY_PORT | Port to 256 deep spy memory |
| A2 | PR | RESET_SPY_MEM_EXY_COUNTER | Reset pointer to Exy spy memory |
| A4 | RO | SPY_MEM_ET_PORT | Port to 256 deep spy memory |
| A6 | PR | RESET_SPY_MEM_ET_COUNTER | Reset pointer to Et spy memory |

## STATUS_REG

A register containing module status information.

| Bit | Description |
| --- | --- |
| 0 | Lock status of DLL |

## CONTROL_REG

A register containing  modul controls.

| Bit | Description |
| --- | --- |
| 0-3 | reserved |
| 4 | Enable spy mode |

## PULSE_REG

A register containing pulsed module controls. Writing zero has no effect.

| Bit | Description |
| --- | --- |
| 0 | Module reset |
| 1 | Simulate short broadcast (reset playback/spy pointers) |
| 10 | Simulate Level1Accept |

## CFG_MASK_REG

Enable configuration download of processors via VME.

| Bit | Description |
| --- | --- |
| 0 | Enable jet processor configuration |
| 4 | Enable configuration of input module R |
| 5 | Enable configuration of input module S |
| 6 | Enable configuration of input module T |
| 7 | Enable configuration of input module U |

## FPGA_RESET_REG

Clear configuration of processors.

| Bit | Description |
| --- | --- |
| 0 | Reset jet processor configuration |
| 4 | Reset configuration of input module R |
| 5 | Reset configuration of input module S |
| 6 | Reset configuration of input module T |
| 7 | Reset configuration of input module U |

## DONE_REG

Indicate configuration status (DONE line) of processors.

| Bit | Description |
| --- | --- |
| 0 | jet processor configuration done |
| 4 | configuration of input module R done |
| 5 | configuration of input module S done |
| 6 | configuration of input module T done |
| 7 | configuration of input module U done |

## TTC_STATUS_REG

Read out TTCrx status and data via I2C. Data are valid only if neither busy nor error bit are asserted.

| Bit | Description |
| --- | --- |
| 0-7 | Data read from I2C |
| 13 | I2C busy |
| 14 | I2C error |

## TTC_CONTROL_REG

Control TTCrx chip via I2C. Access to this register is allowed only if I2C controller not busy. See 4.7. To read/write TTCrx registers specify sub-address and data to be written. For TTCrx address map see TTCrx reference manual.

| Bit | Description |
| --- | --- |
| 0-7 | Data to be written to I2C |
| 8-12 | I2C sub-address |
| 13 | Write control ‘1’= write to I2C device |
| 15 | Reset I2C controller |

## READ_REQUEST_DELAY_REG

A register containing the latency correction for the READ_REQUEST signal . The READ_REQUEST signal initiates DAQ readout on all Input-FPGAs and on the Sum Processor.

| Bit | Description |
| --- | --- |
| 0-5 | Latency correction, up to 63 ticks |

## ROC_SLICE_REG

The slice count register contains the number of consecutive slices which will be read out after a L1A signal. The maximum number is five.

| Bit | Description |
| --- | --- |
| 0-2 | Slice count, up to 5 ticks |

## BC_PRESET_REG

A register containing the timing correction for the local bunch crossing counter. This value is used to pre-set the bunch counter upon reception of a BC_RESET signal from the TTCrx.

| Bit | Description |
| --- | --- |
| 0-11 | Bunch counter preset |

## SPY_MEM_EXY_PORT

A register serving as a port to the 256 deep spy memory. After each register read operation the pointer to the spy memory location is incremented. Pointer reset via 4.13.

| 0-7 | Ex data. Copy of the data sent to the energy sum merger, quad linear encoded. |
| --- | --- |
| 8-15 | Ey data. Copy of the data sent to the energy sum merger, quad linear encoded. |

## RESET_SPY_MEM_EXY_COUNTER

Reset for spy memory pointer (pulse register)

| Bit | Description |
| --- | --- |
| 0 | Set spy memory pointer to 0 location |

## SPY_MEM_ET_PORT

A register serving as a port to the 256 deep spy memory. After each register read operation the pointer to the spy memory location is incremented. Pointer reset via 4.15.

| 0-7 | Et data. Copy of the data sent to the energy sum merger, quad linear encoded. |
| --- | --- |
| 8 | Odd parity bit as sent to merger along with the data |

## RESET_SPY_MEM_EXY_COUNTER

Reset for spy memory pointer (pulse register)

| Bit | Description |
| --- | --- |
| 0 | Set spy memory pointer to 0 location |

# Input FPGA register map

Registers relating to the FPGA as a whole are starting at subaddress 0. Input channel related registers are located from Address 0x1000.

| VME Address (Hex) | Type | Name | Description |
| --- | --- | --- | --- |
| 0000 | RO | VERSION_REG | Firmware version number |
| 0002 | RO | STATUS_REG | Status register (currently not in use) |
| 0004 | RW | CONTROL_REG | Control register |
| 0006 | W | PULSE_REG | Pulse register |
| 0008 | RW | THRESHOLD_LOW | Jet element low threshold register (Ex,Ey) |
| 000A | RW | THRESHOLD_HIGH | Jet element high threshold register(Et) |
| 0010 | RW | PLAYSPY_REG | R/W port for playback and spy, auto increment |
| 0011 | PR | RESET_PLAYSPY_COUNTER | Reset pointer to playback/spy memory location |
| 1000 |  |  | Sub-address space for input channels. 24 individual channels spaced 0x40. |

## PULSE_REG

A register containing  pulsed module controls. Writing zero has no effect.

| Bit | Description |
| --- | --- |
| 0 | Reset link error counters |
| 1 | Reset Parity Error counters |
| 2 | Reset Test Pattern Error Counters |
| 4 | Reset counter for Playback/Spy Memory access |

## CONTROL_REG

A register containing module controls.

| Bit | Description |
| --- | --- |
| 0 | Enable playback mode |
| 1 | Enable spy mode |

## THRESHOLD_LOW , THRESHOLD_HIGH

A set of 2 control registers (10 bit wide) setting a low threshold, applied to jet elements before calculation of missing energy, and a high threshold applied to jet elements to be summed up in the total transverse energy summation tree.

## PLAYSPY_REG

A port to the playback/spy memory array of 24 words × 256 deep. Pointer to the playback/spy memory location is incremented after each write or read operation. Looping over data words first, then depth. Pointer reset via 5.5. Raw data words are transmitted, the meaning of which is different in playback or spy mode.

| Bit | Description |
| --- | --- |
| 0-9 | Playback / spy raw data word |

## RESET_PLAYSPY_COUNTER

A register to reset the pointer to the playback/spy memory.

| Bit | Description |
| --- | --- |
| 0 | Reset pointer to playback/spy memory |

## Channel Space

For each input channel (electromagnetic or hadronic) a set of registers is mapped. Please note that some registers may carry jet element control functions. Those registers are found in every other channel space only (even or odd).

| VME Address (Hex) | Type | Name | Description |
| --- | --- | --- | --- |
| 0 | RO | Ch_Status | Status Register |
| 2 | RW | Ch_control | Control Register |
| 4 | RO | Ch_Link_err | Link error counter, 12 bit saturating counter |
| 6 | RO | Ch_Par_err | Parity error counter, 12 bit saturating counter |
| 8 | RO | Ch_Test_Pat_err | Test pattern detection error counter, 12 bit |
| A | RW | Ch_mult | Ex/Ey coefficient on even/odd channel |

### Ch_status

| Bit | Description |
| --- | --- |
| 0 | Link error bit (current status) |

### Ch_control

| Bit | Description |
| --- | --- |
| 0 | Phase control bit (+1/2 tick) |
| 2:1 | Delay control bit field (+ n ticks) |
| 3 | Mask bit : ‘1’ = off |

### Ch_Link_err

12-bit saturating counter. Counts the transitions of the link status line. Large values indicate instable rather than broken link. The link error bit in the status register needs to be read out to determine current link status.

### Ch_Par_err

12-bit saturating counters. Counts parity errors.

### Ch_Test_Pat_err

12-bit saturating counters. Counts pattern errors. Used for firmware based analysis of linear ramp patterns generated by the upstream module.

### Ch_mult

12-bit coefficient for calculation of EX, EY from the jet element which is built from one electromagnetic and one corresponding hadronic channel. CH_mult of electromagnetic channels (even channel numbers) is the X-coefficient, CH_mult of hadronic channels (odd) is the Y-coefficient.

Change log:

2005-12-09 Change wording of SystemACE register description to reflect hardware modifications on JEM1.3 agreed with L1Calo community on Dec. 08 (16-bit access to ACE only).

2005-12-05 Extract from FDRed JEM specifications and implement corrections in response to FDR comments. Add some register descriptions. Add SystemACE

Consecutive read accesses are used to read spy memory.