Level-1 Calorimeter Trigger Phase-I Upgrades

Report of the ATLAS Calorimeter & Trigger Working Group

C. Bohm, A. Cerqueira, D. Damazio, P. Farthouat, N. Gee, L. Hervas, S. Hillier,

J. Linnemann, F. Lanni, B. Mansoulie, H.C Schulz-Coulon, S. Rajagoapalan, S. Silverstein,

O. Solovyanov, A. Straessner

the ATLAS Calorimeter & Trigger Working Group

ABSTRACT: The Calorimeter & Trigger (C&T) working group has been mandated by the LAr, Tile and TDAQ Project Leaders in December 2011 to understand the implications of upgrading the calorimeter level-1 system with higher granularity and higher precision information, and with the possibility of deploying more powerful algorithms for the electron selection and the rejection of background jets during the ATLAS operations in Phase-I (L '3 ⇥1034cm-2s-1) and beyond. This reports summarizes the conclusions of the C&T working group, laying out the foundation of the future upgrade projects, reviewing the topics agreed by the communities of the three subsystems, and also pointing out where further studies are needed to reach a consensual design.

EDMS Doc.: ATU-GE-ER-0002 v.1 EDMS Id: 1241011 URL: https://edms.cern.ch/document/1241011/1

| 6 | Contents |  |
| --- | --- | --- |
|  | Executive Summary | 3 |
| 8 | 1. Introduction and mandate of the working group | 7 |
|  | 2. Architecture options | 7 |
| 10 12 | 3. Granularity of the EM ’Super-cells’ 3.1 “Super-Cell” conﬁguration in the barrel E.M. calorimeter 3.2 “Super-Cell” conﬁguration in the end-cap E.M. calorimeter 3.3 “Super-Cell” summary tables for the E.M. calorimeters | 8 8 9 9 |
| 14 16 18 | 4. Granularity in the Hadronic Calorimeter 4.1 Hadronic veto studies for the Level-1 electron triggers 4.2 Inclusive jet and multijet trigger studies 4.3 Conﬁguration in the TileCal 4.4 Conﬁguration in the End-Cap hadronic calorimeter | 16 16 17 19 20 |
|  | 5. Granularity in the Forward Calorimeter | 23 |
| 20 | 6. Usage of the Tile D-layer | 27 |
|  | 7. Resolution and dynamic range required | 27 |
| 22 | 8. Front-End digitization and analog-digital interfaces | 28 |
|  | 9. Bandwidth and organisation of optical ﬁbers to USA15 | 32 |
| 24 | 10. Architecture of the LDPS | 32 |
| 26 28 | 11. Bandwidth and organization of optical ﬁbers from DPS to FEX 11.1 E.M. data to eFEX 11.2 E.M. data to jFEX 11.3 Fiber duplication and routing | 35 35 35 35 |
| 30 32 34 36 | 12. Architecture of the Level-1 Calorimeter Trigger system in Phase-I. Routing of the legacy analogue signals and of the digital “Super-Cells” 12.1 FEX architectures 12.2 Hardware Constraints on eFEX and jFEX partitioning 12.3 Routing of digital EM supercells to FEX systems 12.4 Routing of legacy analog hadronic towers to FEX systems 12.4.1 Datapath through the nMCM 12.4.2 Datapath through a TileCal Trigger Digitizer Board in USA-15 | 35 36 36 36 37 37 37 |
|  | 13. Latency | 40 |
| 38 | 14. Compatibility with Phase-II | 43 |

–1–

15. Conclusions 43

40

–2–

Executive Summary

42 After the planned LHC shutdown LS2 in 2018, the luminosity in ATLAS is expected to exceed the design luminosity by a factor of three. The Level-1 (L1) trigger rate will not be allowed to exceed

44 100 kHz, since it will require major upgrades of the front-end readout of most subsystems. This imposes a major constraint on the performance of the Level-1 trigger and single lepton thresholds

46 would have to be raised at levels that impact extensively physics acceptance in processes at the electroweak scale.

48 The usage of higher granularity information at L1 from the calorimeters has been proven by preliminary studies to be an important mitigating factor since it increases the background jet rejec

50 tion. There are several possible options to upgrade the L1 calorimeter trigger, and a working group

52 was formed by the Project Leaders of Liquid Argon (LAr), TileCal and TDAQ to analyze them, identify their major issues and risks, and come up with a set of recommendations for the architecture.

54 This section of the report summarizes the ﬁndings and the recommendations of the C&T working group (WG), together with open points of attention, which needs to be addressed and answered

56 before the approval of the upgrade projects of the systems involved.

Findings and recommendations

58

• Need of upgrades vs. ’do-nothing’ option

Refs. [1]-[2] show the predicted rates expeted during operations after LS2. Preliminary

60 shower shape algorithms using increased granularity calorimeter information have demonstrated an effective reduction of the rates from QCD jets, while maintaining fully efﬁcient se

62 lectivity of electrons. Risks related to the installation and the commissioning of the proposed system by the end of the LS2 shutdown are moderate (see next bullet). We acknowledge the

64 need for the Phase-I upgrades and we recommend to move forward to the deﬁnition and the speciﬁcation of the system.

66

• Digital only vs. Analog and Digital

We have evaluated the pros and the cons of implementing the Phase-I upgrade digital only

68 system. The existing Level-1 calorimeter (L1Calo) trigger required an extensive period of time between construction, installation and commissioning. A similar effort for a fully new

70 L1Calo would not be compatible with the duration of the LS2 shutdown and risks would be too high. We fully endorse the decision to develop a solution that allows ATLAS to maintain

72 the existing L1Calo system and to integrate it with the new higher granularity and higher precision digital elements. This strategy allows for better ﬂexibility in the future phases, for

74 possibilities of staging, and for an “adiabatic” integration in ATLAS.

• Front-End: separation between functionalities

76 Operating side-by-side in the Front-End crates the “legacy” analog trigger builder boards (TBBs) and the “new” LAr Trigger Digitizer Boards (LTDBs) seems feasible. We recommend

78 to use this approach as baseline design for the E.M. calorimeters. In the HEC and FCAL detectors the trigger driver boards (TDB) are relatively sparesely populated and both options

80 (single mixed analog/digital board or two separate boards) are possible. The overall complexity of the front-end system in the two cases will determine what is the optimal solution.

82 Such a decision can in any case be made at a later stage.

• Granularity of the E.M. calorimeters

–3–

84

Ref. [3] summarizes the conﬁguration and the granularity of all the LAr calorimeters. The trigger information from the E.M. calorimeters could be based on the so-called “1441” scheme

86

(i.e. the number of Super-Cells element in a trigger tower). Early studies using a shower shape variable showed some sensible but not signiﬁcant advantage over the “1141” scheme.

88

The 1441 scheme guarantees better ﬂexibility, it allows to implement in future different and better shower shaper algorithms, and it may adapt better to the needs of the calorimeter

90

trigger in Phase-II. For these reasons we recommend it as baseline option.

• Granularity of the Forward Calorimeters

92

Performance studies should determine how much the forward jet and missing transverse energy trigger will degrade. These studies are not completed at this stage.

94

Anticipating the harsh conditions expected for both Phase-I and Phase-II upgrades in the Forward regions, a scheme has been developed to increase the granularity in the FCal1 and

96

FCal2 modules. There are no critical technical challenges, and the impact on the overall costs of the project is very modest. The recommendation of this working group is to include

98

the high granularity FCal scheme in the baseline design. However, we also recommend to prioritize the simulation performance studies for the next steps of the upgrade project

100 approval.

• Granularity of the Hadronic Calorimeter

102 The hadronic calorimeter granularity in h, f is ﬁxed by the construction both in the TileCal and in the LAr HEC modules. Studies on jet trigger turn-on curves and on the hadronic veto in

104 the electron triggers convincingly indicate that there is no clear beneﬁt in using the hadronic calorimeter longitudinal segmentation rather than the the full hadronic trigger tower. We don’t

106 recommend to use layer information from the hadronic calorimeters at L1.

• TileCal D-layer

108 Dedicated D-layer outputs are available at the trigger patch panel in USA-15. The reason for this was their potential use for supporting muon identiﬁcation. The value of conﬁning the

110 muon track to a given D-cell or via the time of ﬂight between the D-cell and registration in the muon detector is probably marginal (' 2mm would be needed) but needs to be studied.

112 The advantage of using D-cell data to sharpen trigger turn-on curves has been shown to be negligible (see above). Unless new contradictory results are produced it seems unlikely that

114 the D-cells will be used to enhance the trigger.

• Latency budget

116 The routing of the new digital signals introduce extra–latency. Preliminary estimates are consistent with the ATLAS overall latency budget: dedicated high rate tests with the existing AT

118 LAS detectors have shown the feasibility of operating the ATLAS detector with an increased L1A latency of 18-20 ticks. The new calorimeter trigger system ﬁts into the budget. How

120 ever, it is not yet well understood what are the safety margins, in particular because of the constraints on the L1A by the SCT detector. It is therefore very important to further develop

122 the analysis of the overall system, possibly to conﬁrm some of the assumptions made so far with measurements, as soon as the ﬁrst hardware prototype modules become available.

124 A full analysis of the latency budget should be among the top priorities of the systems in preparation of their TDRs.

126

• Compatibility with Phase-II upgrades

–4–

The architecture proposed seems consistent with the current plans for the upgrade of the

128 trigger system in Phase-II. In case of a dual hardware-based ﬁrst level trigger (L0/L1) the system under consideration would naturally mature in the future Level-0 calorimeter trigger.

130 In case ATLAS will decide for a single Level-1 trigger with extended latency and rates, the system installed in Phase-I could form the core of the trigger input stage for clustering and for

132 the e/g and jet/E processors. The usage of full granularity information from the calorimeter RODs will build on it, but its performance beneﬁts have to be fully investigated.

134 Open questions and points of attention

• ADC resolution and dynamic range

136 Using 12-bit ADC resolution seems a feasible option and should be adopted as baseline design. Truncation to 10-bits may be implemented in the Digital Processing System (DPS) in

138 case it is needed to guarantee the total data throughput. Deﬁnition of the least signiﬁcant bit (LSB), or equivalently of the MSB should be derived by

140 physics and performance studies. For algorithms to be implemented in the eFEX there is a consistent advantage of having 64 MeV (or 256 MeV as second choice) precision data. For

142 jet and MET trigger studies, it seems that it 256 MeV (or 512 MeV) would be acceptable. Further investigation is required. While the decision is not a show-stopper, it may impact the

144 design of the TileCal-Feature Extractor (FEX) data interface (see below).

• Organization of the ﬁbers between the DPS and the FEX systems

146 To provide sufﬁcient ’environment’ for sliding window algorithms, each FEX system will typically require two copies of each link from the DPS’s FPGAs. The preferred solution is to

148 have four parallel sets of optical transmitters for each output, two for each FEX. For technical reasons, the eFEX partitioning may require an additional two-fold duplication of some digital

150 input links. It is not yet decided how this additional duplication should be implemented. Considering the early stage of development, no recommendation is given by this working

152 group at this moment, on how this information duplication should be implemented technically. Speciﬁcation of the interfaces, organization of the ﬁbers, data bandwidth and transmission

154 protocols (see also next item) are being evaluated.

• Data protocols: BCMuX and alternatives

156 A time multiplexing protocol (BCMuX) is used in the existing L1 data-path between the Pre-Processor and the Cluster Processor modules that allows two channels to be read out on

158 the same link in two consecutive bunch crossings, essentially halving the number of links necessary.

160 The currently planned eFEX architecture relies on such a reduced link count between the DPS and eFEX, and the L1Calo group strongly favor the implementation of a similar BC

162 Mux scheme for this purpose. Motivations for this include the fact that BCMuX is simple to implement, has a small protocol overhead, and has a ﬁxed, low latency of a single bunch

164 crossing. The BCMuX protocol assumes the use of a BC identiﬁcation (BCID) algorithm based on a

166 digital ﬁlter applied to the raw ADC data over several BCs, followed by a peak ﬁnder that identiﬁes BCs where the ﬁlter output is a maximum. By using a peak-ﬁnder based BCID

168 algorithm, no two consecutive bunch crossings can be non-zero for any channel. Therefore, two neighbouring Super Cells will at most report two non-zero values over two consecutive

170 bunch crossings.

–5–