JEM test programme (Mainz)

VME:		no dedicated test programme, VME is used to test all other aspects of JEM functionality.

Timing:	No TTC hardware available at Mainz (TTCvi missing). Neither expertise nor

human resources available to set up a test system. Currently the system timing is derived from a single DSS clocking the JEM at 40 MHz.

RTDP (sum):	all de-serialisers fed by link replicator module via twist’n flat cable.

Sum data spied on via spy memory / VME before being driven to the backplane. Board-level output path will be checked with an oscilloscope with a binary counter pattern only.

Software required: test vector generation/transmission as well as spy readout and vector analysis

RTDP (jet):	Any tests on jet specific hardware/firmware will require effort from Stockholm. For reason of data integrity the inter-JEM communication will require an additional programmable clock on the receiving end (main processor). This is available from the TTCrx only. First tests might be possible by using a separate delay unit and a micro-backplane feeding the  FIO lines back to the same JEM.

DAQ&ROI	No dedicated test hardware available. Data could be sunk	into DSS, if required. Requires completion of the ROC firmware and some on-line and analysis software.

Configurator:	At current the FPGAs can be configured via VME only. No further work will be done on configuration hardware. On future iterations CP-compatible cofigurator hardware will be used. No tests anticipated.

CAN:	since there won’t be any compatible hardware on the final modules, no effort will be made to complete and test the DCS interface.